CORPORATION OF THE TOWNSHIP OF ESQUIMALT

Municipal Hall, 1229 Esquimalt Road, Victoria, B.C.  V9A 3P1

Engineering & Public Works

Telephone: (250) 414-7108     Fax: (250) 414-7160

MEMORANDUM

DATE:		February 14, 2023

TO:		Development Services

FROM:	Richard Syrett

SUBJECT:	Application for Development Permit and Development Variance Permit at 314 Uganda Avenue

Engineering has completed a preliminary review of the proposed development and have the following comments:

- Completion of Works and Services

According to Bylaw 2175, including all Schedules, the developer may be required to provide all Works and Services up to the road centerline. All Works and Services that are required to be constructed and installed at the expense of the Owner shall be constructed before the Approving Officer approves the development unless the Owner:

  - Deposits with the Municipality a Security Deposit in the amount of 120% of the estimated construction cost, and

  - Enters into a Servicing Agreement with the Municipality

- Serviceability

A preliminary review reveals that the subject property is connected to the Municipal Sewer and Drain Systems, but no information could be found regarding age or condition. Dye test results indicate that the subject property does have the benefit of a Municipal Storm Service Connection. The proposed development is to have Services as per Bylaw 2175 (Subdivision and Development Control Bylaw), including, but not limited to, new sewer and drain service connections if required. New curb and gutter along the frontage may also be required. Applicant to confirm water servicing requirements with the City of Victoria.

- Engineering

No information pertaining to the age or condition of the existing sewer and drain service connections were found. A Test and Clear of the existing sewer and drain service connections will be required to determine if the existing can be used, or new connections are required The Applicant is responsible for retaining the services of a qualified professional for the design and construction supervision of all Works and Services, including construction cost, engineering fees, administrative fees, and as indicated in Bylaw 2175.

- Additional comments

Additional review comments will be provided when detailed engineering drawings are submitted.

Sincerely,

Richard Syrett, AScT

Engineering Technologist III

/tmp/in/input.doc